Specification & Firm Price Quotation:-To provide a decorative and protective resin bonded exterior weatherproof wall treatment to                   elevations of To include access equipment and site clearance.

Preparation:		To include some or all of the following elements where necessary. Scarifying to remove all loose and friable deposits. Biocide cleansing to inhibit any further algae contamination. Hammer Testing of any existing cement rendered smooth/pebble or tyrolen surface to locate hollow or blown areas. Re-rendering with Unibond building adhesive. Our operatives will use their best endeavours to restore surface defects to blend with the overall existing wall surface although on some surfaces these repairs may be detectable in certain lights. Bag Rubbing where necessary the brick surface with a modified P.V.A. sand and cement slurry to restore mortar joints and the    spalling brick face.  This unique process retains the character of broken bond appearance. Epoxy Resin will be utilised to attend to perpend mortar/brick interface cracking caused by thermal expansion and/or undue pressure to prevent further opening plus re-bedding of all loose brickwork.

# Application: 		To include Masking with wax paper and tape all windows, doors and  itemised fixtures & fittings. Sheeting with thermo- plastic decking all walk-ways and low level roofing. Hessian coverage for all ajacent foliage and shrubs. Pressure application of the two pack system “Tex-gard” in the required film thickness of not less than one kilo per square metre having first applied a saturation resin key coat.

# Completion: 		Site Clearance of all debris and final inspection of the completed  workmanship by the Supervising Crew Operative in conjunction with the property owners whereupon the satisfaction note will be presented for approval.

# Guarantee: Comprehensive cover for a period of Twenty years from the date of completion on all masonry type surfaces with thirty-five years proof of performance plus BBA support. Optional independent insurance backed warranty also available. Specially formulated for hand application, a five-litre colour coded touch repair canister retained on site for minor alterations.

- Total Cost          	  £____________		Including VAT

- Limited Offer Valid for acceptance before or until 	______________________

- Terms:       One third by initial payment.     Final balance due upon completion.

To accept this firm price limited offer please sign and return this quotation retaining a file copy

Signed	___________________		Date__________________      File Copy